# Phase 2 Day 1 – ESP32 IoT Alerting Setup Foundation

## 1. Objective of the Day

The objective of Phase 2 Day 1 was to prepare the ESP32 foundation for the IoT alerting stage of the AI Intruder Detection System.

The main focus was not to implement full Wi-Fi or Telegram alerting yet, but to first verify that the ESP32 board is correctly detected, programmed, and able to control basic output devices such as an LED and buzzer.

By the end of the day, the ESP32 was expected to:

Detect properly in Arduino IDE
Accept code uploads successfully
Control an external LED
Control a buzzer
Respond to Serial Monitor commands

## 2. Work Completed

### 2.1 ESP32 Board Support Installation

The first task was to install ESP32 board support in the Arduino IDE.

The ESP32 board manager URL was added in:

File → Preferences → Additional Boards Manager URLs

The following URL was used:

https://espressif.github.io/arduino-esp32/package_esp32_index.json

After adding the URL, the Boards Manager was opened from:

Tools → Board → Boards Manager

Then esp32 by Espressif Systems was searched and installed successfully.

After installation, ESP32 board options became available in Arduino IDE.

## 3. ESP32 Board and Port Selection

After installing ESP32 board support, the ESP32 was connected to the laptop using a USB cable.

The selected board was:

ESP32 Dev Module

Initially, the ESP32 COM port was not visible in Arduino IDE. This created a connection issue, because without the COM port the board could not be programmed.

## 4. Challenge Faced: ESP32 COM Port Not Showing

### Problem

When the ESP32 was connected to the laptop, the port number was not showing in Arduino IDE under:

Tools → Port

This indicated that the laptop was not properly detecting the ESP32 USB-to-serial interface.

### Cause

The issue was related to the missing USB-to-UART driver. The ESP32 board required a USB bridge driver for proper communication between the laptop and ESP32.

### Solution

The Silicon Labs USB to UART Bridge VCP driver was installed.

After installing the driver, the ESP32 COM port became visible in Arduino IDE.

The ESP32 was successfully detected on:

COM4

This confirmed that the driver issue was solved and the laptop could now communicate with the ESP32.

## 5. ESP32 Blink Test

After the ESP32 was detected, a basic blink test was uploaded to verify that the board could accept code and execute it.

The initial blink code used GPIO 2:

#define LED_PIN 2

void setup() {
  pinMode(LED_PIN, OUTPUT);
}

void loop() {
  digitalWrite(LED_PIN, HIGH);
  delay(1000);

  digitalWrite(LED_PIN, LOW);
  delay(1000);
}

The code uploaded successfully. The upload output confirmed that the ESP32 was connected and programmed correctly.

Important upload confirmations included:

Connected to ESP32 on COM4
Hash of data verified
Hard resetting via RTS pin...

This proved that the ESP32 itself was working properly.

## 6. Challenge Faced: GPIO 2 LED Not Blinking

### Problem

Although the code uploaded successfully, the LED did not blink when GPIO 2 was used.

### Cause

The onboard LED on this ESP32 board was either not connected to GPIO 2 or was not available as a programmable onboard LED. ESP32 boards can vary depending on manufacturer and model, so the onboard LED pin is not always the same.

### Solution

An external LED was connected and tested using a different GPIO pin.

After testing, the LED worked successfully on:

GPIO 23

So, the LED pin was changed from GPIO 2 to GPIO 23 for the remaining tests.

Updated LED pin:

#define LED_PIN 23

This solved the LED blinking issue.

## 7. External LED and Buzzer Setup

After verifying that the external LED worked on GPIO 23, the LED and buzzer were connected to the ESP32 for output testing.

Final pin configuration used:

LED Pin    → GPIO 23
Buzzer Pin → GPIO 22
GND        → Common Ground

The wiring was done as follows:

ESP32 GPIO 23 → LED long leg
LED short leg → 220Ω resistor → GND

ESP32 GPIO 22 → Buzzer positive
Buzzer negative → GND

A common ground connection was maintained between the LED and buzzer circuit.

## 8. LED and Buzzer Test

The LED and buzzer were tested together using ESP32 GPIO pins.

Test code:

#define LED_PIN 23
#define BUZZER_PIN 22

void setup() {
  pinMode(LED_PIN, OUTPUT);
  pinMode(BUZZER_PIN, OUTPUT);
}

void loop() {
  digitalWrite(LED_PIN, HIGH);
  digitalWrite(BUZZER_PIN, HIGH);
  delay(1000);

  digitalWrite(LED_PIN, LOW);
  digitalWrite(BUZZER_PIN, LOW);
  delay(1000);
}

### Output Observed

The test was successful.

LED turned ON and buzzer turned ON for 1 second
LED turned OFF and buzzer turned OFF for 1 second
The cycle repeated continuously

This confirmed that both output devices were working properly with the ESP32.

## 9. ESP32 Serial Command Mode Test

After testing continuous LED and buzzer operation, command mode was implemented.

The purpose of command mode was to make the ESP32 respond to Serial Monitor input.

Command logic:

Send 1 → LED and buzzer ON
Send 0 → LED and buzzer OFF

Command mode code:

#define LED_PIN 23
#define BUZZER_PIN 22

void setup() {
  pinMode(LED_PIN, OUTPUT);
  pinMode(BUZZER_PIN, OUTPUT);

  Serial.begin(9600);
}

void loop() {
  if (Serial.available() > 0) {
    char data = Serial.read();

    if (data == '1') {
      digitalWrite(LED_PIN, HIGH);
      digitalWrite(BUZZER_PIN, HIGH);
    }
    else if (data == '0') {
      digitalWrite(LED_PIN, LOW);
      digitalWrite(BUZZER_PIN, LOW);
    }
  }
}

The Serial Monitor was opened and the baud rate was set to:

9600

Testing was performed by sending commands manually.

### Output Observed

When 1 was sent:

LED turned ON
Buzzer turned ON

When 0 was sent:

LED turned OFF
Buzzer turned OFF

This confirmed that ESP32 command mode worked successfully.

## 10. Challenges and Solutions

| Challenge | Cause | Solution |
| --- | --- | --- |
| ESP32 COM port was not showing | Missing USB-to-UART driver | Installed Silicon Labs USB to UART Bridge VCP driver |
| GPIO 2 LED was not blinking | ESP32 board LED pin variation / onboard LED not mapped to GPIO 2 | Used external LED on GPIO 23 |
| Needed reliable output pins | Original test pins were changed after practical testing | Final LED pin set to GPIO 23 and buzzer pin set to GPIO 22 |

## 11. Final Pin Configuration

The final working ESP32 pin configuration for Phase 2 Day 1 was:

LED    → GPIO 23
Buzzer → GPIO 22
GND    → Common Ground

This pin configuration will be used for the next ESP32 IoT alerting steps.

## 12. Result

Phase 2 Day 1 was completed successfully.

The following tasks were achieved:

ESP32 board support installed in Arduino IDE
ESP32 detected successfully after driver installation
ESP32 connected on COM4
Code upload verified successfully
External LED tested successfully on GPIO 23
Buzzer tested successfully on GPIO 22
LED and buzzer tested together
ESP32 Serial Monitor command mode tested successfully

## 13. Conclusion

Phase 2 Day 1 successfully established the ESP32 foundation for IoT alerting.

Although two challenges were faced, both were solved practically. The ESP32 COM port issue was fixed by installing the Silicon Labs USB to UART Bridge VCP driver. The LED issue was solved by changing the LED output pin from GPIO 2 to GPIO 23.

The ESP32 is now able to control the LED and buzzer and respond to Serial Monitor commands. This confirms that the hardware foundation is ready for the next stage.

## Phase 2 Day 2 – ESP32 WiFi Foundation and Status Alert Testing

## 1. Objective

The objective of Phase 2 Day 2 was to test the WiFi capability of the ESP32 and prepare it for future IoT-based alerting.

After completing Phase 2 Day 1, the ESP32 hardware foundation was ready. The board was detected in Arduino IDE, external LED and buzzer were tested, and Serial Monitor command mode was verified.

On Day 2, the main goal was to confirm that the ESP32 can:

Connect to a WiFi network
Display WiFi status in Serial Monitor
Print its local IP address
Use LED as WiFi status indicator
Use buzzer as success/failure indicator
Handle WiFi failure condition properly

This step is important before adding Telegram alerts or cloud-based IoT notifications.

## 2. Work Completed

The following tasks were completed on Phase 2 Day 2:

Created Phase 2 Day 2 project folder structure
Tested basic ESP32 WiFi connection
Verified ESP32 IP address output in Serial Monitor
Tested WiFi status indication using LED and buzzer
Tested WiFi failure condition using wrong password
Saved Serial Command Mode as backup code
Prepared Day 2 report notes

## 3. Hardware Components Used

| Component | Purpose |
| --- | --- |
| ESP32 WiFi + Bluetooth Board | Main IoT microcontroller |
| External LED | Visual WiFi status indicator |
| 220Ω Resistor | Current limiting for LED |
| Active Buzzer | Audio indication for success/failure |
| Breadboard | Temporary circuit connection |
| Jumper Wires | Wiring connections |
| USB Cable | Programming and power supply |

## 4. Software and Tools Used

| Software / Tool | Purpose |
| --- | --- |
| Arduino IDE | Programming ESP32 |
| ESP32 Board Support Package | ESP32 board programming support |
| WiFi.h Library | ESP32 WiFi connectivity |
| Serial Monitor | WiFi status and IP address monitoring |
| Silicon Labs USB to UART Bridge VCP Driver | ESP32 COM port detection |

## 5. Final Pin Configuration

The pin configuration used from Day 1 was continued on Day 2.

LED Pin    → GPIO 23
Buzzer Pin → GPIO 22
GND        → Common Ground
Baud Rate  → 9600
Board      → ESP32 Dev Module

This configuration was used because GPIO 23 worked reliably for the external LED during Day 1 testing.

## 6. Folder Structure Created

A separate Day 2 folder structure was prepared to keep the project organized.

Suggested structure:

Phase-2-ESP32-IoT-Alerting/
│
├── Day-1-ESP32-Setup/
│
├── Day-2-WiFi-Foundation/
│   ├── esp32_wifi_test/
│   │   └── esp32_wifi_test.ino
│   │
│   ├── esp32_wifi_led_buzzer_status/
│   │   └── esp32_wifi_led_buzzer_status.ino
│   │
│   └── notes/
│       └── phase2_day2_notes.md
│
└── backup_codes/
    └── esp32_command_mode.ino

This structure helps separate basic setup, WiFi testing, status indication testing, and backup codes.

## 7. Task 1 – Project Folder Setup

The first task was to create and organize the Phase 2 Day 2 files.

The following folders were prepared:

Day-2-WiFi-Foundation
esp32_wifi_test
esp32_wifi_led_buzzer_status
notes
backup_codes

This made the project easier to manage and prepared it for future IoT alerting code.

### Result

Task 1 completed successfully.
Day 2 project folder structure was created.

## 8. Task 2 – ESP32 WiFi Connection Test

The second task was to connect the ESP32 to a WiFi network using the built-in ESP32 WiFi library.

The WiFi.h library was used.

### Code Used

#include <WiFi.h>

const char* ssid = "YOUR_WIFI_NAME";
const char* password = "YOUR_WIFI_PASSWORD";

void setup() {
  Serial.begin(9600);
  delay(1000);

  Serial.println("Connecting to WiFi...");
  WiFi.begin(ssid, password);

  while (WiFi.status() != WL_CONNECTED) {
    delay(500);
    Serial.print(".");
  }

  Serial.println("");
  Serial.println("WiFi Connected Successfully!");
  Serial.print("ESP32 IP Address: ");
  Serial.println(WiFi.localIP());
}

void loop() {
}

The WiFi name and password were entered in the code before uploading.

### Serial Monitor Settings

Baud Rate: 9600

### Expected Output

Connecting to WiFi...
.....
WiFi Connected Successfully!
ESP32 IP Address: 192.168.x.x

### Observed Result

The ESP32 connected successfully to the WiFi network and printed the local IP address in the Serial Monitor.

### Result

Task 2 completed successfully.
ESP32 WiFi connection was verified.

## 9. Task 3 – WiFi Status Indication Using LED and Buzzer

After confirming basic WiFi connection, the next task was to add physical status indication using the external LED and buzzer.

The purpose was to make the ESP32 show connection progress and result without depending only on the Serial Monitor.

### Status Logic

| Condition | LED Output | Buzzer Output |
| --- | --- | --- |
| Connecting to WiFi | LED blinks | OFF |
| WiFi connected successfully | LED stays ON | One short beep |
| WiFi connection failed | LED stops | Three short beeps |

### Code Used

#include <WiFi.h>

#define LED_PIN 23
#define BUZZER_PIN 22

const char* ssid = "YOUR_WIFI_NAME";
const char* password = "YOUR_WIFI_PASSWORD";

void setup() {
  Serial.begin(9600);

  pinMode(LED_PIN, OUTPUT);
  pinMode(BUZZER_PIN, OUTPUT);

  digitalWrite(LED_PIN, LOW);
  digitalWrite(BUZZER_PIN, LOW);

  Serial.println("Connecting to WiFi...");
  WiFi.begin(ssid, password);

  int attempt = 0;

  while (WiFi.status() != WL_CONNECTED && attempt < 20) {
    digitalWrite(LED_PIN, HIGH);
    delay(200);
    digitalWrite(LED_PIN, LOW);
    delay(300);

    Serial.print(".");
    attempt++;
  }

  if (WiFi.status() == WL_CONNECTED) {
    Serial.println("");
    Serial.println("WiFi Connected Successfully!");
    Serial.print("ESP32 IP Address: ");
    Serial.println(WiFi.localIP());

    digitalWrite(LED_PIN, HIGH);

    digitalWrite(BUZZER_PIN, HIGH);
    delay(300);
    digitalWrite(BUZZER_PIN, LOW);
  } 
  else {
    Serial.println("");
    Serial.println("WiFi Connection Failed!");

    for (int i = 0; i < 3; i++) {
      digitalWrite(BUZZER_PIN, HIGH);
      delay(200);
      digitalWrite(BUZZER_PIN, LOW);
      delay(200);
    }
  }
}

void loop() {
}

### Observed Result

The LED blinked while the ESP32 was attempting to connect to WiFi.
After successful connection, the LED stayed ON and the buzzer gave one short beep.

The Serial Monitor displayed the WiFi connection success message and the ESP32 local IP address.

### Result

Task 3 completed successfully.
LED and buzzer WiFi status indication worked correctly.

## 10. Task 4 – WiFi Failure Case Testing

The fourth task was to test how the ESP32 behaves when WiFi connection fails.

This was done by intentionally entering a wrong WiFi password.

### Purpose of Failure Test

This test was important because IoT systems should not only work during success conditions. They must also give clear feedback during failure conditions.

A failure condition may happen due to:

Wrong WiFi password
Router unavailable
Weak WiFi signal
Network disconnected
SSID changed

### Test Method

The WiFi password in the code was temporarily changed to an incorrect password.

Example:

const char* password = "WRONG_PASSWORD";

The code was uploaded again and Serial Monitor was opened at 9600 baud rate.

### Expected Output

Connecting to WiFi...
....................
WiFi Connection Failed!

### Expected Hardware Output

LED blinks during WiFi connection attempts
Buzzer beeps three times after connection failure

### Observed Result

The ESP32 attempted to connect to WiFi.
After the connection attempt limit was reached, the Serial Monitor displayed the failure message.
The buzzer beeped three times to indicate failure.

After the failure test, the correct WiFi password was restored and the ESP32 connected successfully again.

### Result

Task 4 completed successfully.
WiFi failure condition was tested and verified.

## 11. Task 5 – Serial Command Mode Backup

The Day 1 Serial Command Mode code was saved as a backup.

Backup file:

esp32_command_mode.ino

### Why Backup Was Important

This backup code is useful for quick hardware testing if future WiFi or Telegram code creates issues.

It can verify:

ESP32 upload is working
Serial communication is working
LED is working
Buzzer is working
GPIO pins are working

### Backup Code Reference

#define LED_PIN 23
#define BUZZER_PIN 22

void setup() {
  pinMode(LED_PIN, OUTPUT);
  pinMode(BUZZER_PIN, OUTPUT);

  Serial.begin(9600);
}

void loop() {
  if (Serial.available() > 0) {
    char data = Serial.read();

    if (data == '1') {
      digitalWrite(LED_PIN, HIGH);
      digitalWrite(BUZZER_PIN, HIGH);
    }
    else if (data == '0') {
      digitalWrite(LED_PIN, LOW);
      digitalWrite(BUZZER_PIN, LOW);
    }
  }
}

### Result

Task 5 completed successfully.
Serial Command Mode backup was saved.

## 12. Challenges Faced and Solutions

| Challenge | Reason | Solution | Status |
| --- | --- | --- | --- |
| Needed WiFi connection verification | ESP32 IoT alerting requires stable WiFi first | Tested WiFi using WiFi.h library | Solved |
| Needed physical WiFi status indication | Serial Monitor alone is not enough for hardware feedback | Used LED and buzzer for connection status | Solved |
| Needed failure condition testing | IoT systems must handle connection failure | Tested wrong password case | Solved |
| Needed hardware fallback code | Future IoT code may create debugging difficulty | Saved Serial Command Mode as backup | Solved |

## 13. Final Output Summary

At the end of Phase 2 Day 2, the ESP32 successfully performed the following:

Connected to WiFi
Printed local IP address
Displayed connection progress in Serial Monitor
Blinked LED during connection attempt
Kept LED ON after successful connection
Beeped buzzer once after WiFi success
Detected WiFi failure condition
Beeped buzzer three times after WiFi failure
Reconnected successfully after restoring correct password

## 14. Final Working Configuration

Board: ESP32 Dev Module
Port: COM4
WiFi Library: WiFi.h
Baud Rate: 9600
LED Pin: GPIO 23
Buzzer Pin: GPIO 22
Status LED: Active during WiFi status
Buzzer: Success/failure audio indicator

## 15. Importance of Day 2

Phase 2 Day 2 was an important step because it moved the ESP32 from simple local output control to network connectivity.

Day 1 proved:

ESP32 can be programmed
LED and buzzer can be controlled
Serial command mode works

Day 2 proved:

ESP32 can connect to WiFi
ESP32 can report network status
ESP32 can indicate success and failure physically
ESP32 is ready for IoT alert integration

This prepares the system for the next stage, where WiFi-based notifications can be implemented.

## Day 3 – Python AI Detection + ESP32 Serial Integration

## 1. Objective

The objective of Phase 2 Day 3 was to integrate the existing Python OpenCV AI intruder detection system with the ESP32 hardware alert system using serial communication.

In Phase 2 Day 1, the ESP32 hardware foundation was completed. The ESP32 was detected in Arduino IDE, external LED and buzzer were tested, and Serial Monitor command mode was verified.

In Phase 2 Day 2, the ESP32 WiFi foundation was tested. The ESP32 successfully connected to WiFi, displayed its IP address, and used LED and buzzer indicators for WiFi success and failure.

On Day 3, the main goal was to connect the AI detection logic with the ESP32 alert hardware, so that the ESP32 responds automatically when the Python system detects an intruder.

## 2. Day 3 Main Goal

The target system flow for Day 3 was:

Laptop Camera
→ Python OpenCV Motion Detection
→ Face Detection
→ Motion + Face Decision Logic
→ Python sends command to ESP32 through Serial
→ ESP32 turns ON LED + Buzzer
→ Safe condition returns
→ Python sends OFF command
→ ESP32 turns OFF LED + Buzzer

The aim was not to add Telegram or email alerting yet.
The focus was purely on local AI-to-ESP32 integration.

## 3. Work Completed

The following tasks were completed on Phase 2 Day 3:

✅ ESP32 kept in Serial Command Mode
✅ Python pyserial library installed
✅ ESP32 COM port detected through Python
✅ Python serial test file created
✅ Python successfully controlled ESP32 LED and buzzer
✅ ESP32 serial function added to main AI detection code
✅ Motion + face condition used for alert triggering
✅ Full AI-to-ESP32 integration tested successfully
✅ Day 3 notes prepared

## 4. Hardware Components Used

| Component | Purpose |
| --- | --- |
| ESP32 WiFi + Bluetooth Board | Main alert controller |
| External LED | Visual intruder alert |
| Active Buzzer | Audio intruder alert |
| 220Ω Resistor | LED current protection |
| Breadboard | Temporary circuit setup |
| Jumper Wires | Hardware connections |
| USB Cable | ESP32 programming and serial communication |
| Laptop Camera | Video input for AI detection |

## 5. Software and Libraries Used

| Software / Library | Purpose |
| --- | --- |
| Python | Main AI detection programming |
| OpenCV | Motion detection, face detection, webcam processing |
| pyserial | Python-to-ESP32 serial communication |
| Arduino IDE | ESP32 code upload |
| ESP32 Serial Command Mode Code | Receives commands from Python |
| Haar Cascade Classifier | Face detection |
| Windows Terminal / VS Code Terminal | Python script execution |

## 6. Final Working Pin Configuration

The working ESP32 pin configuration from Day 1 was continued.

LED Pin    → GPIO 23
Buzzer Pin → GPIO 22
Baud Rate  → 9600
Port       → COM4
Board      → ESP32 Dev Module

This configuration was used because GPIO 23 worked reliably for the external LED and GPIO 22 worked for the buzzer.

# 7. Task 1 – ESP32 Serial Command Mode

## 7.1 Purpose

The first task was to keep the ESP32 in Serial Command Mode.

In this mode, ESP32 waits for serial input from Python:

| Serial Command | ESP32 Output |
| --- | --- |
| 1 | LED ON + Buzzer ON |
| 0 | LED OFF + Buzzer OFF |

This made the ESP32 ready to receive commands from the Python AI system.

## 7.2 ESP32 Code Used

#define LED_PIN 23
#define BUZZER_PIN 22

void setup() {
  pinMode(LED_PIN, OUTPUT);
  pinMode(BUZZER_PIN, OUTPUT);

  digitalWrite(LED_PIN, LOW);
  digitalWrite(BUZZER_PIN, LOW);

  Serial.begin(9600);
}

void loop() {
  if (Serial.available() > 0) {
    char data = Serial.read();

    if (data == '1') {
      digitalWrite(LED_PIN, HIGH);
      digitalWrite(BUZZER_PIN, HIGH);
    }
    else if (data == '0') {
      digitalWrite(LED_PIN, LOW);
      digitalWrite(BUZZER_PIN, LOW);
    }
  }
}

## 7.3 Result

The ESP32 command mode worked successfully.

Manual testing through Serial Monitor confirmed:

Send 1 → LED + Buzzer ON
Send 0 → LED + Buzzer OFF

This confirmed that the ESP32 was ready to receive alert commands from Python.

# 8. Task 2 – Python Serial Library Installation

## 8.1 Purpose

The second task was to install the Python serial communication library.

The library used was:

pyserial

This library allows Python to communicate with ESP32 through the USB COM port.

## 8.2 Command Used

pip install pyserial

After installation, the ESP32 COM port was checked using:

python -m serial.tools.list_ports

## 8.3 Result

Python successfully detected the ESP32 COM port.

The ESP32 was available on:

COM4

This confirmed that Python could access the ESP32 through serial communication.

# 9. Task 3 – Python ESP32 Serial Test

## 9.1 Purpose

Before integrating the ESP32 with the main AI detection code, a separate Python serial test file was created.

This was done to verify that Python could directly control the ESP32 LED and buzzer.

## 9.2 Test File Name

python_esp32_serial_test.py

## 9.3 Python Serial Test Code

import serial
import time

ESP32_PORT = "COM4"
BAUD_RATE = 9600

try:
    esp32 = serial.Serial(ESP32_PORT, BAUD_RATE, timeout=1)
    time.sleep(2)

    print("ESP32 connected successfully.")

    print("Turning alert ON...")
    esp32.write(b'1')
    time.sleep(2)

    print("Turning alert OFF...")
    esp32.write(b'0')
    time.sleep(1)

    esp32.close()
    print("Test completed.")

except Exception as e:
    print("ESP32 serial test failed:", e)

## 9.4 Expected Output

ESP32 connected successfully.
Turning alert ON...
LED + Buzzer ON
Turning alert OFF...
LED + Buzzer OFF
Test completed.

## 9.5 Result

The Python serial test was completed successfully.

Python sent:

1 → ESP32 alert ON
0 → ESP32 alert OFF

The ESP32 LED and buzzer responded correctly.

This confirmed that Python-to-ESP32 serial communication was working.

# 10. Task 4 – ESP32 Serial Function Added to AI Detection Code

## 10.1 Purpose

After confirming direct Python control of ESP32, the serial communication logic was added to the main Python OpenCV intruder detection code.

The aim was to create a reusable function that can send alert commands to ESP32 whenever the AI logic confirms an intruder.

## 10.2 Required Imports

import serial
import time

## 10.3 ESP32 Serial Connection Code

ESP32_PORT = "COM4"
BAUD_RATE = 9600

try:
    esp32 = serial.Serial(ESP32_PORT, BAUD_RATE, timeout=1)
    time.sleep(2)
    print("ESP32 connected successfully.")
except Exception as e:
    esp32 = None
    print("ESP32 connection failed:", e)

## 10.4 ESP32 Alert Function

def send_to_esp32(signal):
    if esp32 is not None:
        if signal == 1:
            esp32.write(b'1')
            print("ESP32 Alert ON")
        else:
            esp32.write(b'0')
            print("ESP32 Alert OFF")
    else:
        print("ESP32 not connected. Alert skipped.")

## 10.5 Result

The send_to_esp32() function was added successfully.

This function made the main AI detection code cleaner, because the alert command could now be called using:

send_to_esp32(1)   # Alert ON
send_to_esp32(0)   # Alert OFF

# 11. Task 5 – Intruder-Based ESP32 Alert Trigger

## 11.1 Purpose

The next task was to trigger ESP32 only when an intruder was actually confirmed.

The system should not trigger on only motion or only face detection.

## 11.2 Alert Condition Used

The alert condition was:

if recent_motion and len(faces) > 0:

This means the alert turns ON only when:

Recent motion detected
+
At least one face detected
=
Intruder confirmed

## 11.3 Why This Condition Is Important

Motion detection alone can trigger false alerts due to:

Lighting changes
Moving curtains
Shadows
Camera noise
Background movement

Face detection alone can trigger incorrectly due to:

Photos
Screens
Posters
Static faces in frame
False positive detections

Combining motion and face detection makes the system more reliable.

## 11.4 State-Based Alert Control

The system uses a variable:

system_active = False

This variable prevents repeated ON commands from being sent continuously.

When an intruder is detected and the system is OFF:

send_to_esp32(1)
system_active = True

When the intruder condition becomes false and the system is ON:

send_to_esp32(0)
system_active = False

This makes the alert system stable and avoids unnecessary repeated serial commands.

## 11.5 Result

The ESP32 alert triggered correctly only when both motion and face were detected.

Test results:

| Condition | Result |
| --- | --- |
| No motion + no face | SAFE |
| Motion only | ESP32 stayed OFF |
| Face only | ESP32 did not repeatedly trigger |
| Motion + face | ESP32 LED + buzzer turned ON |
| Person left frame | ESP32 LED + buzzer turned OFF |

# 12. Task 6 – Full System Test

## 12.1 Purpose

The full system test verified the complete Day 3 integration pipeline.

## 12.2 Full Pipeline Tested

Camera captures video
→ Python detects motion
→ Python detects face
→ Python confirms intruder
→ Python sends '1' to ESP32
→ ESP32 turns ON LED and buzzer
→ Intruder condition ends
→ Python sends '0' to ESP32
→ ESP32 turns OFF LED and buzzer

## 12.3 Expected Terminal Output

During intruder detection:

Intruder Detected!
ESP32 Alert ON

When safe condition returned:

ESP32 Alert OFF

## 12.4 Result

The full system test was completed successfully.

The laptop camera, Python AI logic, and ESP32 hardware alert worked together as one integrated system.

This completed the first full AI-to-ESP32 alert flow for VisionGuard Phase 2.

# 13. Challenges Faced

## 13.1 COM Port Access Conflict

### Problem

Python and Arduino Serial Monitor cannot use the same COM port at the same time.

If Arduino Serial Monitor remains open, Python may fail to connect to ESP32.

### Solution

Before running the Python AI detection code, Arduino Serial Monitor was closed.

### Result

Python successfully accessed COM4 and communicated with ESP32.

## 13.2 Safe Serial Connection Handling

### Problem

If ESP32 is not connected, Python may fail while creating the serial object.

### Solution

A try-except block was used:

try:
    esp32 = serial.Serial(ESP32_PORT, BAUD_RATE, timeout=1)
    time.sleep(2)
except Exception as e:
    esp32 = None

### Result

The program does not immediately crash if ESP32 connection fails. Instead, it prints an error message and skips ESP32 alerts.

## 13.3 Repeated Alert Commands

### Problem

The OpenCV loop runs many times per second. Without control logic, Python would send repeated 1 commands continuously while the intruder remains in frame.

### Solution

A state variable was used:

system_active = False

This ensures the ON command is sent only once when the alert begins, and the OFF command is sent only once when the alert ends.

### Result

The ESP32 alert became stable and did not receive unnecessary repeated commands.

## 13.4 False Trigger Possibility

### Problem

Motion alone or face detection alone can create false alerts.

### Solution

The alert condition requires both:

recent_motion == True
faces detected > 0

### Result

The alert becomes more reliable compared to using only motion or only face detection.

# 14. Potential Changes and Improvements

## 14.1 Rename Arduino Comments to ESP32

Some older comments in the code still mention Arduino because the code originally came from Phase 1.

### Current Comment Style

Arduino Alert
Sends byte '1' to Arduino
Arduino LED/Buzzer

### Recommended Change

Replace with:

ESP32 Alert
Sends byte '1' to ESP32
ESP32 LED/Buzzer

This will make the code documentation consistent with Phase 2.

## 14.2 Add Safe Serial Close Check

At the end of the code, instead of directly writing:

esp32.close()

use:

if esp32 is not None:
    esp32.close()

This prevents errors if ESP32 was not connected.

## 14.3 Add First Camera Frame Validation

After:

ret, prev_frame = cap.read()

add:

if not ret:
    print("Camera not detected. Please check webcam connection.")
    cap.release()
    cv2.destroyAllWindows()
    if esp32 is not None:
        esp32.close()
    exit()

This prevents a crash if the webcam fails to provide the first frame.

## 14.4 Add Alert Duration Control

Currently, the alert remains ON while the intruder condition is true and turns OFF when the system becomes safe.

A future version can use a fixed alert duration:

Intruder detected → LED/Buzzer ON for 2 seconds → OFF automatically

This can prevent long buzzer activation.

## 14.5 Add Cooldown for ESP32 Alerts

The image capture cooldown is already present, but a separate ESP32 alert cooldown can be added.

Example:

alert_cooldown = 10
last_alert_time = 0

This can prevent repeated buzzer alerts during continuous detection.

## 14.6 Add WiFi + Telegram Alert

Since Day 2 already verified WiFi, the next major improvement can be:

Intruder Detected
→ ESP32 LED + Buzzer ON
→ ESP32 sends Telegram alert

This will make the system a real IoT alert system.

## 14.7 Add WiFi Reconnection Logic

In future IoT alerting, ESP32 should be able to reconnect automatically if WiFi disconnects.

Possible logic:

If WiFi disconnected:
    Try reconnecting
    Blink LED
    Buzzer warning if connection fails

## 14.8 Store Captured Images with Timestamp

Currently, images are saved using a counter:

intruder_0.jpg
intruder_1.jpg

A better future format would be:

intruder_2026-05-09_14-30-15.jpg

This makes captured evidence easier to organize.

## 14.9 Add Logging File

A log file can be created to store intruder events:

Date
Time
Motion Score
Face Count
Image Saved Path
ESP32 Alert Status

Example log:

2026-05-09 14:30:15 | ALERT | Motion Score: 12540 | Faces: 1 | ESP32 ON

## 14.10 Add Configuration Section

Instead of hardcoding values throughout the code, all important settings can be placed at the top:

ESP32_PORT = "COM4"
BAUD_RATE = 9600
MOTION_THRESHOLD = 7000
MOTION_HOLD_TIME = 2
CAPTURE_COOLDOWN = 5

This makes tuning easier.

## 14.11 Add Graceful Exit Using Try-Finally

A future version can wrap the main loop in try-finally.

This ensures that even if an error happens:

Camera is released
ESP32 alert turns OFF
Serial connection closes
OpenCV windows close

This is important for hardware safety.

# 15. Final Output Summary

At the end of Day 3, the system successfully achieved:

✅ ESP32 ready in Serial Command Mode
✅ Python pyserial installed and tested
✅ Python detected ESP32 on COM4
✅ Python controlled ESP32 LED and buzzer
✅ ESP32 serial function added to AI code
✅ Motion + face condition connected to ESP32 alert
✅ Full AI-to-ESP32 pipeline tested
✅ ESP32 alert turned ON during intruder detection
✅ ESP32 alert turned OFF when safe condition returned

## Day 4 – ESP32 Telegram IoT Notification Preparation

## 1. Objective

The objective of Phase 2 Day 4 was to move VisionGuard from local ESP32 alerting toward remote IoT notification alerting.

In earlier days of Phase 2:

Day 1 → ESP32 hardware setup completed
Day 2 → ESP32 WiFi foundation completed
Day 3 → Python AI detection integrated with ESP32 using serial communication

On Day 4, the main goal was to test whether ESP32 could send a remote alert message using Telegram Bot API.

The focus was not yet full AI-triggered Telegram automation.
The goal was to prepare and verify the Telegram notification foundation.

## 2. Day 4 Main Goal

The target flow for Day 4 was:

ESP32
→ Connect to WiFi
→ Send Telegram test message
→ Receive message on Telegram app
→ Confirm success using LED and buzzer
→ Test failure cases
→ Protect secret credentials before GitHub upload

This step prepares the system for the next stage where the Python AI detection system will trigger ESP32, and ESP32 will send remote Telegram alerts.

## 3. Work Completed

The following tasks were completed on Phase 2 Day 4:

✅ Day 4 project folder created
✅ Telegram bot created using BotFather
✅ Bot token generated and saved privately
✅ Telegram Chat ID collected using getUpdates API
✅ UniversalTelegramBot library installed
✅ ArduinoJson library installed
✅ ESP32 Telegram test message sent successfully
✅ Telegram message received on mobile app
✅ LED and buzzer confirmation added
✅ WiFi failure case tested
✅ Wrong bot token failure case tested
✅ Secret credentials protected before GitHub upload

## 4. Hardware Components Used

| Component | Purpose |
| --- | --- |
| ESP32 WiFi + Bluetooth Board | Main IoT microcontroller |
| External LED | Visual status indication |
| Active Buzzer | Audio confirmation and failure warning |
| 220Ω Resistor | LED current protection |
| Breadboard | Temporary hardware setup |
| Jumper Wires | Circuit connections |
| USB Cable | ESP32 programming and power |
| Laptop | Arduino IDE programming |
| Mobile Phone | Telegram message receiving |

## 5. Software and Libraries Used

| Software / Library | Purpose |
| --- | --- |
| Arduino IDE | Programming ESP32 |
| ESP32 Board Support Package | ESP32 development support |
| WiFi.h | ESP32 WiFi connection |
| WiFiClientSecure.h | Secure HTTPS communication |
| UniversalTelegramBot.h | Telegram Bot API communication |
| ArduinoJson | JSON handling required by Telegram library |
| Telegram App | Receiving bot alert messages |
| BotFather | Creating Telegram bot |

## 6. Final Working Configuration

The working pin configuration from previous Phase 2 tasks was continued.

Board      : ESP32 Dev Module
Port       : COM4
Baud Rate  : 9600
LED Pin    : GPIO 23
Buzzer Pin : GPIO 22
WiFi       : 2.4 GHz Network

The LED and buzzer were used for status indication:

| Condition | LED Output | Buzzer Output |
| --- | --- | --- |
| Connecting to WiFi | LED blinks | OFF |
| Telegram message sent successfully | LED blinks once | Buzzer beeps once |
| WiFi or Telegram failure | LED stops / does not confirm | Buzzer beeps three times |

# 7. Task 1 – Day 4 Folder Setup

## 7.1 Purpose

A separate folder was created for Day 4 to keep Telegram IoT notification files organized.

## 7.2 Suggested Folder Structure

Phase-2-ESP32-IoT-Alerting/
│
├── Day-4-Telegram-IoT-Notification/
│   ├── esp32_telegram_test/
│   │   └── esp32_telegram_test.ino
│   │
│   ├── esp32_wifi_telegram_alert/
│   │   └── esp32_wifi_telegram_alert.ino
│   │
│   └── notes/
│       └── phase2_day4_notes.md

## 7.3 Outcome

The Day 4 structure was created successfully.
This helped separate Telegram notification work from earlier WiFi and serial integration tasks.

# 8. Task 2 – Telegram Bot Creation

## 8.1 Purpose

A Telegram bot was needed so that ESP32 could send alert messages to the user through Telegram.

## 8.2 Method

The bot was created using BotFather on Telegram.

Commands used:

/start
/newbot

BotFather asked for:

Bot name
Bot username

After successful creation, BotFather generated a Bot Token.

## 8.3 Important Security Point

The bot token was treated as private information.

The real token was not shared publicly and was not uploaded to GitHub.

## 8.4 Outcome

Telegram bot was created successfully and the bot token was generated.

# 9. Task 3 – Telegram Chat ID Collection

## 9.1 Purpose

To send a message to a specific Telegram account, ESP32 needs the receiver’s Chat ID.

## 9.2 Method

First, a message was sent to the created bot:

/start
hello

Then the Telegram Bot API getUpdates method was used in the browser:

https://api.telegram.org/botYOUR_BOT_TOKEN/getUpdates

Initially, the response showed:

{"ok":true,"result":[]}

## 9.3 Challenge Faced

The result was empty because the bot had not received any message yet.

## 9.4 Solution

The created bot was opened directly, and a message was sent to it.
After refreshing the getUpdates URL, the Chat ID appeared inside the response.

Example structure:

"chat":{"id":123456789}

## 9.5 Outcome

The Chat ID was successfully collected and saved privately.

# 10. Task 4 – Required Library Installation

## 10.1 Purpose

ESP32 needed Telegram-related libraries to communicate with the Telegram Bot API.

## 10.2 Libraries Installed

In Arduino IDE:

Sketch → Include Library → Manage Libraries

The following libraries were installed:

UniversalTelegramBot
ArduinoJson

## 10.3 Why These Libraries Are Needed

| Library | Purpose |
| --- | --- |
| UniversalTelegramBot | Sends and receives Telegram bot messages |
| ArduinoJson | Handles JSON data used by Telegram API |
| WiFiClientSecure | Enables HTTPS connection from ESP32 |

## 10.4 Outcome

Required Telegram libraries were installed successfully.

# 11. Task 5 – ESP32 Telegram Test Message

## 11.1 Purpose

The goal of this task was to verify that ESP32 could connect to WiFi and send a message to Telegram.

## 11.2 Code Used

#include <WiFi.h>
#include <WiFiClientSecure.h>
#include <UniversalTelegramBot.h>

// WiFi details
const char* ssid = "YOUR_WIFI_NAME";
const char* password = "YOUR_WIFI_PASSWORD";

// Telegram bot details
#define BOT_TOKEN "YOUR_BOT_TOKEN"
#define CHAT_ID "YOUR_CHAT_ID"

WiFiClientSecure client;
UniversalTelegramBot bot(BOT_TOKEN, client);

void setup() {
  Serial.begin(9600);
  delay(1000);

  Serial.println("Connecting to WiFi...");
  WiFi.begin(ssid, password);

  while (WiFi.status() != WL_CONNECTED) {
    delay(500);
    Serial.print(".");
  }

  Serial.println("");
  Serial.println("WiFi Connected Successfully!");
  Serial.print("ESP32 IP Address: ");
  Serial.println(WiFi.localIP());

  // Required for HTTPS connection
  client.setInsecure();

  Serial.println("Sending Telegram message...");

  bool messageSent = bot.sendMessage(
    CHAT_ID,
    "VisionGuard ESP32 Telegram test successful.",
    ""
  );

  if (messageSent) {
    Serial.println("Telegram message sent successfully.");
  } else {
    Serial.println("Telegram message failed.");
  }
}

void loop() {
}

## 11.3 Explanation of Important Code Sections

### WiFi Library

#include <WiFi.h>

This allows ESP32 to connect to a WiFi network.

### Secure Client Library

#include <WiFiClientSecure.h>

Telegram API works over HTTPS, so a secure client is required.

### Telegram Library

#include <UniversalTelegramBot.h>

This library simplifies sending Telegram messages from ESP32.

### Telegram Bot Object

WiFiClientSecure client;
UniversalTelegramBot bot(BOT_TOKEN, client);

This creates a secure client and connects it with the Telegram bot token.

### HTTPS Certificate Handling

client.setInsecure();

This allows ESP32 to make HTTPS requests without manually adding Telegram’s SSL certificate.

For a student prototype, this is acceptable.
For production-level systems, certificate validation should be handled more securely.

### Sending Telegram Message

bool messageSent = bot.sendMessage(
  CHAT_ID,
  "VisionGuard ESP32 Telegram test successful.",
  ""
);

This sends a test message to the Telegram chat ID.

## 11.4 Expected Output

Serial Monitor:

WiFi Connected Successfully!
ESP32 IP Address: 192.168.x.x
Sending Telegram message...
Telegram message sent successfully.

Telegram app:

VisionGuard ESP32 Telegram test successful.

## 11.5 Outcome

The Telegram message was successfully received from the ESP32.

This confirmed:

ESP32 WiFi connection working
Telegram Bot API working
Bot token and Chat ID correct
ESP32 can send remote IoT notification

This was the major milestone of Day 4.

# 12. Task 6 – LED and Buzzer Confirmation

## 12.1 Purpose

After confirming Telegram message delivery, LED and buzzer confirmation were added.

This gave the system both:

Remote confirmation → Telegram message
Local confirmation  → LED and buzzer

## 12.2 Code Used

#include <WiFi.h>
#include <WiFiClientSecure.h>
#include <UniversalTelegramBot.h>

#define LED_PIN 23
#define BUZZER_PIN 22

const char* ssid = "YOUR_WIFI_NAME";
const char* password = "YOUR_WIFI_PASSWORD";

#define BOT_TOKEN "YOUR_BOT_TOKEN"
#define CHAT_ID "YOUR_CHAT_ID"

WiFiClientSecure client;
UniversalTelegramBot bot(BOT_TOKEN, client);

void successAlert() {
  digitalWrite(LED_PIN, HIGH);
  digitalWrite(BUZZER_PIN, HIGH);
  delay(300);
  digitalWrite(LED_PIN, LOW);
  digitalWrite(BUZZER_PIN, LOW);
}

void failureAlert() {
  for (int i = 0; i < 3; i++) {
    digitalWrite(BUZZER_PIN, HIGH);
    delay(200);
    digitalWrite(BUZZER_PIN, LOW);
    delay(200);
  }
}

void setup() {
  Serial.begin(9600);
  delay(1000);

  pinMode(LED_PIN, OUTPUT);
  pinMode(BUZZER_PIN, OUTPUT);

  digitalWrite(LED_PIN, LOW);
  digitalWrite(BUZZER_PIN, LOW);

  Serial.println("Connecting to WiFi...");
  WiFi.begin(ssid, password);

  int attempt = 0;

  while (WiFi.status() != WL_CONNECTED && attempt < 20) {
    digitalWrite(LED_PIN, HIGH);
    delay(200);
    digitalWrite(LED_PIN, LOW);
    delay(300);

    Serial.print(".");
    attempt++;
  }

  if (WiFi.status() == WL_CONNECTED) {
    Serial.println("");
    Serial.println("WiFi Connected Successfully!");
    Serial.print("ESP32 IP Address: ");
    Serial.println(WiFi.localIP());

    client.setInsecure();

    Serial.println("Sending Telegram message...");

    bool messageSent = bot.sendMessage(
      CHAT_ID,
      "VisionGuard ESP32 Telegram test successful.",
      ""
    );

    if (messageSent) {
      Serial.println("Telegram message sent successfully.");
      successAlert();
    } else {
      Serial.println("Telegram message failed.");
      failureAlert();
    }
  } else {
    Serial.println("");
    Serial.println("WiFi Connection Failed!");
    failureAlert();
  }
}

void loop() {
}

## 12.3 Explanation of LED and Buzzer Logic

### During WiFi Connection

while (WiFi.status() != WL_CONNECTED && attempt < 20) {
  digitalWrite(LED_PIN, HIGH);
  delay(200);
  digitalWrite(LED_PIN, LOW);
  delay(300);

  Serial.print(".");
  attempt++;
}

The LED blinks while ESP32 is trying to connect to WiFi.

If the LED blinks three times, it means ESP32 needed around three connection attempts before WiFi connected.

### Success Alert Function

void successAlert() {
  digitalWrite(LED_PIN, HIGH);
  digitalWrite(BUZZER_PIN, HIGH);
  delay(300);
  digitalWrite(LED_PIN, LOW);
  digitalWrite(BUZZER_PIN, LOW);
}

This function runs only when the Telegram message is sent successfully.

Output:

LED blinks once
Buzzer beeps once

### Failure Alert Function

void failureAlert() {
  for (int i = 0; i < 3; i++) {
    digitalWrite(BUZZER_PIN, HIGH);
    delay(200);
    digitalWrite(BUZZER_PIN, LOW);
    delay(200);
  }
}

This function runs when WiFi connection fails or Telegram message sending fails.

Output:

Buzzer beeps three times

## 12.4 Outcome

The LED and buzzer confirmation worked successfully.

Observed behavior:

LED blinked during WiFi connection attempts
Telegram message was received
LED blinked once at the end
Buzzer beeped once at the end

This confirmed that the local and remote notification indicators were working together.

# 13. Task 7 – Failure Case Testing

## 13.1 Purpose

Failure testing was performed to check how the system behaves when connection or credentials are incorrect.

IoT systems should not only work in success conditions.
They should also give clear output during failure conditions.

## 13.2 Failure Case 1 – Wrong WiFi Password

### Test

The WiFi password was temporarily changed to an incorrect value.

const char* password = "WRONG_PASSWORD";

### Expected Result

WiFi Connection Failed!
Buzzer beeps three times
Telegram message is not sent

### Outcome

The ESP32 failed to connect to WiFi as expected.
The buzzer gave warning beeps, confirming failure handling.

## 13.3 Failure Case 2 – Wrong Bot Token

### Test

The bot token was temporarily changed to an incorrect value.

#define BOT_TOKEN "WRONG_BOT_TOKEN"

### Expected Result

WiFi Connected Successfully!
Sending Telegram message...
Telegram message failed.
Buzzer beeps three times

### Outcome

WiFi connected successfully, but Telegram message sending failed.
The failure was detected and the buzzer warning was triggered.

## 13.4 Final Restoration

After failure testing, correct credentials were restored.

The ESP32 again sent the Telegram message successfully.

# 14. Task 8 – Secret Credential Protection

## 14.1 Purpose

The project uses sensitive credentials:

WiFi name
WiFi password
Telegram bot token
Telegram chat ID

These should not be uploaded publicly to GitHub.

## 14.2 Action Taken

Real values were removed from GitHub-ready code and replaced with placeholders.

const char* ssid = "YOUR_WIFI_NAME";
const char* password = "YOUR_WIFI_PASSWORD";

#define BOT_TOKEN "YOUR_BOT_TOKEN"
#define CHAT_ID "YOUR_CHAT_ID"

## 14.3 Security Note for README

A security note should be added:

## Security Note

This project uses WiFi credentials, Telegram Bot Token, and Telegram Chat ID for IoT notification testing.

For security reasons, real credentials are not uploaded to GitHub.  
Before running the code, replace the placeholder values with your own private credentials.

## 14.4 Outcome

The code was made GitHub-safe by removing real private credentials.

# 15. Challenges Faced and Outcomes

| Challenge | Cause | Solution | Outcome |
| --- | --- | --- | --- |
| Telegram bot username already taken | Bot usernames must be unique | Used a unique project-related bot username | Bot created successfully |
| getUpdates returned empty result | Bot had not received any message yet | Sent /start and hello to the bot, then refreshed API URL | Chat ID received successfully |
| Telegram required secure HTTPS communication | Telegram API uses HTTPS | Used WiFiClientSecure and client.setInsecure() | ESP32 could communicate with Telegram |
| Need to confirm message delivery locally | Telegram message alone does not show local hardware status | Added LED and buzzer success indication | LED blinked once and buzzer beeped once after success |
| Need to handle failure cases | Wrong WiFi or wrong bot token can break notification | Added failureAlert() and tested wrong credentials | Buzzer warning worked during failure |
| Sensitive credentials risk | GitHub upload could expose password/token | Replaced real credentials with placeholders | Code became safe for public upload |

# 16. Final Output Summary

At the end of Phase 2 Day 4, the following outputs were achieved:

✅ Telegram bot created
✅ Bot token generated
✅ Chat ID collected
✅ Telegram libraries installed
✅ ESP32 connected to WiFi
✅ ESP32 sent Telegram test message
✅ Telegram message received successfully
✅ LED blinked during WiFi connection
✅ LED and buzzer confirmed success
✅ Wrong WiFi failure tested
✅ Wrong bot token failure tested
✅ Credentials protected for GitHub

## Day 5 – Full AI + ESP32 + Telegram IoT Integration

## 1. Objective

The objective of Phase 2 Day 5 was to complete the full integration between the Python OpenCV AI detection system, ESP32 hardware alert system, and Telegram IoT notification system.

Previous progress:

Phase 2 Day 1 → ESP32 hardware setup completed
Phase 2 Day 2 → ESP32 WiFi foundation tested
Phase 2 Day 3 → Python AI detection integrated with ESP32 using Serial communication
Phase 2 Day 4 → ESP32 Telegram notification tested successfully

On Day 5, the goal was to combine all these individual modules into one complete working prototype.

## 2. Day 5 Main Goal

The main goal of Day 5 was to make the system work in this complete flow:

Laptop Camera
→ Python OpenCV AI Detection
→ Motion + Face Confirmed
→ Python sends Serial command to ESP32
→ ESP32 turns ON LED + Buzzer
→ ESP32 sends Telegram alert
→ Python sends OFF command when safe
→ ESP32 turns OFF LED + Buzzer

This was the first full version where AI detection, hardware alerting, and remote IoT notification worked together.

## 3. Work Completed

The following tasks were completed on Phase 2 Day 5:

✅ Day 5 folder structure created
✅ ESP32 Serial + Telegram alert code prepared
✅ Telegram alert message updated
✅ ESP32 tested using Serial Monitor
✅ Python AI detection code connected with ESP32
✅ Full AI + ESP32 + Telegram system tested
✅ Telegram anti-spam cooldown added
✅ Private credentials protected before GitHub upload
✅ Day 5 notes prepared

## 4. Hardware Components Used

| Component | Purpose |
| --- | --- |
| ESP32 WiFi + Bluetooth Board | Main IoT alert controller |
| External LED | Visual alert indicator |
| Active Buzzer | Audio alert indicator |
| 220Ω Resistor | LED current protection |
| Breadboard | Circuit prototyping |
| Jumper Wires | Hardware connections |
| USB Cable | ESP32 programming and Serial communication |
| Laptop Camera | Video input for intruder detection |
| Mobile Phone | Telegram alert receiving device |

## 5. Software and Libraries Used

| Software / Library | Purpose |
| --- | --- |
| Python | Main AI detection programming |
| OpenCV | Motion detection, face detection, webcam processing |
| pyserial | Python-to-ESP32 Serial communication |
| Arduino IDE | ESP32 programming |
| WiFi.h | ESP32 WiFi connectivity |
| WiFiClientSecure.h | Secure HTTPS communication |
| UniversalTelegramBot.h | Telegram Bot API communication |
| ArduinoJson | JSON support for Telegram library |
| Telegram App | Remote alert receiving platform |

## 6. Final Working Configuration

Board      : ESP32 Dev Module
Port       : COM4
Baud Rate  : 9600
LED Pin    : GPIO 23
Buzzer Pin : GPIO 22
WiFi       : 2.4 GHz Network
Cooldown   : 15 seconds

The ESP32 received Serial commands from Python:

| Command | ESP32 Action |
| --- | --- |
| 1 | LED ON + Buzzer ON + Telegram alert |
| 0 | LED OFF + Buzzer OFF + alert state reset |

# 7. Task 1 – Day 5 Folder Structure

## 7.1 Purpose

A new folder structure was created to keep the full AI + ESP32 + Telegram integration files separate from earlier testing files.

# 8. Task 2 – ESP32 Serial + Telegram Alert Code

## 8.1 Purpose

The ESP32 code was modified so that it could receive commands from Python through Serial communication and send Telegram alerts when an intruder is detected.

## 8.2 ESP32 Logic

Receive '1'
→ Turn ON LED
→ Turn ON buzzer
→ Send Telegram alert

Receive '0'
→ Turn OFF LED
→ Turn OFF buzzer
→ Reset alert state

## 8.3 ESP32 Code

#include <WiFi.h>
#include <WiFiClientSecure.h>
#include <UniversalTelegramBot.h>

#define LED_PIN 23
#define BUZZER_PIN 22

// WiFi credentials
const char* ssid = "YOUR_WIFI_NAME";
const char* password = "YOUR_WIFI_PASSWORD";

// Telegram credentials
#define BOT_TOKEN "YOUR_BOT_TOKEN"
#define CHAT_ID "YOUR_CHAT_ID"

WiFiClientSecure client;
UniversalTelegramBot bot(BOT_TOKEN, client);

bool alertSent = false;

unsigned long lastTelegramTime = 0;
unsigned long telegramCooldown = 15000; // 15 seconds

void connectToWiFi() {
  Serial.println("Connecting to WiFi...");
  WiFi.begin(ssid, password);

  int attempt = 0;

  while (WiFi.status() != WL_CONNECTED && attempt < 20) {
    digitalWrite(LED_PIN, HIGH);
    delay(200);
    digitalWrite(LED_PIN, LOW);
    delay(300);

    Serial.print(".");
    attempt++;
  }

  if (WiFi.status() == WL_CONNECTED) {
    Serial.println("");
    Serial.println("WiFi Connected Successfully!");
    Serial.print("ESP32 IP Address: ");
    Serial.println(WiFi.localIP());

    client.setInsecure();
  } else {
    Serial.println("");
    Serial.println("WiFi Connection Failed!");
  }
}

void successBeep() {
  digitalWrite(BUZZER_PIN, HIGH);
  delay(300);
  digitalWrite(BUZZER_PIN, LOW);
}

void failureBeep() {
  for (int i = 0; i < 3; i++) {
    digitalWrite(BUZZER_PIN, HIGH);
    delay(200);
    digitalWrite(BUZZER_PIN, LOW);
    delay(200);
  }
}

void sendTelegramAlert() {
  if (WiFi.status() == WL_CONNECTED) {
    bool messageSent = bot.sendMessage(
      CHAT_ID,
      "🚨 VisionGuard Alert!\n\nIntruder detected.\nDetection: Motion + Face confirmed.\nAlert Source: ESP32 IoT Module.\nStatus: LED and buzzer activated.",
      ""
    );

    if (messageSent) {
      Serial.println("Telegram alert sent successfully.");
      successBeep();
    } else {
      Serial.println("Telegram alert failed.");
      failureBeep();
    }
  } else {
    Serial.println("WiFi not connected. Telegram alert skipped.");
    failureBeep();
  }
}

void setup() {
  Serial.begin(9600);
  delay(1000);

  pinMode(LED_PIN, OUTPUT);
  pinMode(BUZZER_PIN, OUTPUT);

  digitalWrite(LED_PIN, LOW);
  digitalWrite(BUZZER_PIN, LOW);

  connectToWiFi();

  Serial.println("ESP32 Serial + Telegram Alert System Ready.");
}

void loop() {
  if (Serial.available() > 0) {
    char data = Serial.read();

    if (data == '1') {
      digitalWrite(LED_PIN, HIGH);
      digitalWrite(BUZZER_PIN, HIGH);

      Serial.println("Alert command received from Python.");

      if (!alertSent && millis() - lastTelegramTime > telegramCooldown) {
        sendTelegramAlert();
        alertSent = true;
        lastTelegramTime = millis();
      }
    }

    else if (data == '0') {
      digitalWrite(LED_PIN, LOW);
      digitalWrite(BUZZER_PIN, LOW);

      alertSent = false;

      Serial.println("Safe command received. Alert OFF.");
    }
  }
}

## 8.4 Outcome

The ESP32 code was uploaded successfully.
The board was ready to receive Serial commands and send Telegram alerts.

# 9. Task 3 – Telegram Alert Message Update

## 9.1 Purpose

The Telegram message was changed from a simple test message to a proper project alert message.

## 9.2 Final Telegram Message

🚨 VisionGuard Alert!

Intruder detected.
Detection: Motion + Face confirmed.
Alert Source: ESP32 IoT Module.
Status: LED and buzzer activated.

## 9.3 Why This Message Is Better

The updated message is more professional because it clearly tells:

What happened       → Intruder detected
How it was detected → Motion + Face confirmed
Alert source        → ESP32 IoT Module
System status       → LED and buzzer activated

## 9.4 Outcome

The Telegram alert message became suitable for technical demo, GitHub documentation, and resume presentation.

# 10. Task 4 – ESP32 Standalone Serial Monitor Test

## 10.1 Purpose

Before connecting Python, the ESP32 was tested alone using the Arduino Serial Monitor.

This confirmed whether the ESP32 code could:

Receive Serial input
Control LED and buzzer
Send Telegram alert
Prevent repeated Telegram messages
Reset when safe command is received

## 10.2 Test Method

In Arduino Serial Monitor, baud rate was set to:

9600

Command sent:

1

Expected result:

LED ON
Buzzer ON
Telegram alert received
Serial Monitor shows alert command received

Then command sent:

0

Expected result:

LED OFF
Buzzer OFF
System ready for next alert

## 10.3 Anti-Spam Test

The command 1 was sent multiple times without sending 0.

Expected behavior:

LED and buzzer stay ON
Telegram message should not repeat continuously

After sending 0, command 1 was sent again.

Expected behavior:

New Telegram alert allowed after reset and cooldown

## 10.4 Outcome

ESP32 standalone testing was successful.

The board correctly handled Serial commands, hardware alerts, Telegram messaging, and alert reset logic.

# 11. Task 5 – Python AI Detection Code Connection

## 11.1 Purpose

The Python OpenCV AI detection code was connected with the ESP32 Serial + Telegram code.

In this architecture:

Python handles AI detection
ESP32 handles LED, buzzer, and Telegram notification

This separation makes the system cleaner and more professional.

## 11.2 Python Serial Configuration

import serial
import time

ESP32_PORT = "COM4"
BAUD_RATE = 9600

try:
    esp32 = serial.Serial(ESP32_PORT, BAUD_RATE, timeout=1)
    time.sleep(2)

except Exception as e:
    esp32 = None
    print("ESP32 connection failed:", e)

## 11.3 Python Function to Send Command

def send_to_esp32(signal):
    if esp32 is not None:
        if signal == 1:
            esp32.write(b'1')
            print("ESP32 Alert ON")
        else:
            esp32.write(b'0')
            print("ESP32 Alert OFF")
    else:
        print("ESP32 not connected. Alert skipped.")

## 11.4 AI Alert Condition

The Python alert condition remained:

if recent_motion and len(faces) > 0:

This means the alert is triggered only when:

Recent motion is detected
+
At least one face is detected
=
Intruder confirmed

## 11.5 Python Alert Logic

if recent_motion and len(faces) > 0:

    if not system_active:
        print("Intruder Detected!")

        send_to_esp32(1)

        system_active = True

    if current_time - last_capture_time > cooldown:
        filename = f"captures/intruder_{image_count}.jpg"
        cv2.imwrite(filename, frame)

        image_count += 1
        last_capture_time = current_time

else:

    if system_active:
        send_to_esp32(0)
        system_active = False

## 11.6 Outcome

The Python code was successfully connected with ESP32.

When motion and face were detected together, Python sent 1 to ESP32.
When the condition became safe, Python sent 0 to ESP32.

# 12. Task 6 – Full System Test

## 12.1 Purpose

The full system test verified the complete Day 5 pipeline.

## 12.2 Full Pipeline Tested

Laptop camera captures video
→ Python detects motion
→ Python detects face
→ Python confirms intruder
→ Python sends '1' to ESP32
→ ESP32 turns ON LED and buzzer
→ ESP32 sends Telegram alert
→ Safe condition returns
→ Python sends '0' to ESP32
→ ESP32 turns OFF LED and buzzer

## 12.3 Test Cases

| Test Case | Expected Result |
| --- | --- |
| No motion + no face | System remains SAFE |
| Motion only | No Telegram alert |
| Face only | No Telegram alert |
| Motion + face | LED + buzzer ON + Telegram alert received |
| Same person remains in frame | No Telegram spam |
| Person leaves frame | LED + buzzer OFF |
| Person returns after cooldown | New Telegram alert allowed |

## 12.4 Observed Result

The full system worked successfully.

When motion and face were detected:

Intruder Detected!
ESP32 Alert ON
Telegram alert received

When the person moved away or the condition became safe:

ESP32 Alert OFF

## 12.5 Outcome

Task 6 was completed successfully.

This confirmed the full integration of:

AI Detection + ESP32 Hardware Alert + Telegram IoT Notification

# 13. Task 7 – Cooldown / Anti-Spam Logic

## 13.1 Problem Faced

During testing, a repeated Telegram alert issue was observed.

When the same person moved again after detection, Telegram alerts were triggered again too quickly.

The behavior was:

Python sends '1'
→ ESP32 sends Telegram alert

Python sends '0'
→ ESP32 resets alert state

Movement occurs again
→ Python sends '1'
→ Telegram alert repeats

This caused repeated alerts for the same person or same detection event.

## 13.2 Solution Adde

ESP32-side Telegram cooldown logic was added.

## 13.3 Cooldown Variables

unsigned long lastTelegramTime = 0;
unsigned long telegramCooldown = 15000; // 15 seconds

## 13.4 Cooldown Logic

if (!alertSent && millis() - lastTelegramTime > telegramCooldown) {
  sendTelegramAlert();
  alertSent = true;
  lastTelegramTime = millis();
}

## 13.5 Explanation

lastTelegramTime stores the time when the last Telegram alert was sent.

telegramCooldown defines the minimum time gap between two Telegram alerts.

The value used was:

15000 milliseconds = 15 seconds

So the system allows only one Telegram alert every 15 seconds.

## 13.6 Final Behavior

First detection → Telegram alert sent
Repeated movement within 15 seconds → No Telegram spam
After 15 seconds → New Telegram alert allowed
Safe condition → LED and buzzer turn OFF

## 13.7 Outcome

The repeated Telegram alert issue was solved.

The system now gives reliable alerts without spamming the user.

# 14. Task 8 – Credential Protection

## 14.1 Purpose

The ESP32 Telegram code uses private information:

WiFi name
WiFi password
Telegram Bot Token
Telegram Chat ID

These details should never be uploaded publicly to GitHub.

## 14.2 Action Taken

Real values were removed from GitHub-ready code and replaced with placeholders.

const char* ssid = "YOUR_WIFI_NAME";
const char* password = "YOUR_WIFI_PASSWORD";

#define BOT_TOKEN "YOUR_BOT_TOKEN"
#define CHAT_ID "YOUR_CHAT_ID"

## 14.3 README Security Note

A security note should be included in the README:

## Security Note

This project uses WiFi credentials, Telegram Bot Token, and Telegram Chat ID for IoT notification testing.

For security reasons, real credentials are not uploaded to GitHub. Before running the code, replace the placeholder values with your own private credentials.

## 14.4 Outcome

The code was made safe for GitHub upload.

# 15. Challenges Faced and Solutions

| Challenge | Cause | Solution | Outcome |
| --- | --- | --- | --- |
| Needed to combine AI, ESP32, and Telegram | Earlier modules were tested separately | ESP32 was programmed to receive Serial commands and send Telegram alerts | Full integration completed |
| Needed to avoid Telegram spam | Repeated movement caused repeated alerts | Added 15-second ESP32-side cooldown | Spam reduced successfully |
| ESP32 and Python Serial conflict possibility | Arduino Serial Monitor and Python cannot use COM4 together | Serial Monitor was closed before running Python | Python accessed ESP32 correctly |
| Needed professional alert message | Test message was too basic | Added formatted VisionGuard alert message | Telegram notification became project-ready |
| Secret credential risk | WiFi password, bot token, and chat ID are private | Replaced real credentials with placeholders | GitHub-safe code prepared |
| Need to reset alert cycle | ESP32 should allow new alert after safe condition | alertSent = false added when 0 is received | New alert allowed after reset and cooldown |

# 16. Final Output Summary

At the end of Phase 2 Day 5, the system achieved:

✅ Python AI detection connected with ESP32
✅ ESP32 received Serial commands from Python
✅ ESP32 activated LED and buzzer
✅ ESP32 sent Telegram alert
✅ Motion + face condition triggered alert correctly
✅ Safe condition turned OFF ESP32 alert
✅ Telegram anti-spam cooldown added
✅ One Telegram alert allowed every 15 seconds
✅ Private credentials protected
✅ Full AI + ESP32 + Telegram integration completed